Datafordeler

Dataleverancespecifikation 
RasterHeaders

Version 1.1

2015 03 06

Versions historik

| Dato | Version | Bemærkning |
| --- | --- | --- |
| 26-02-2015 | 1.0 | Første version af dokumentet |
| 06-03-2015 | 1.1 | Kosmetisk rettelse |

Indholdsfortegnelse

Versions historik	2

Introduktion	4

Om dette dokument	4

Raster Headers	5

Introduktion

Om dette dokument

Dette dokument definerer et eksempel på en Raster header.

Raster Headers

Påkrævede headers i raster-filer (eksemplificeret ved GDAL-udskrift af TIFF), jvf. GSTs "Regelsæt for lagring af data i Geodatabanken":

Origin = (636000.000000000000000,6110000.000000000000000)
Pixel Size = (0.160000000000000,-0.160000000000000)
Coordinate System is:
PROJCS["ETRS89 / UTM zone 32N",
GEOGCS["ETRS89",
DATUM["European_Terrestrial_Reference_System_1989",
SPHEROID["GRS 1980",6378137,298.2572221010002,
AUTHORITY["EPSG","7019"]],
AUTHORITY["EPSG","6258"]],
PRIMEM["Greenwich",0],
UNIT["degree",0.0174532925199433],
AUTHORITY["EPSG","4258"]],
PROJECTION["Transverse_Mercator"],
PARAMETER["latitude_of_origin",0],
PARAMETER["central_meridian",9],
PARAMETER["scale_factor",0.9996],
PARAMETER["false_easting",500000],
PARAMETER["false_northing",0],
UNIT["metre",1,
AUTHORITY["EPSG","9001"]],
AUTHORITY["EPSG","25832"]]
Image Structure Metadata:
INTERLEAVE=PIXEL
Corner Coordinates:
Upper Left ( 636000.000, 6110000.000) ( 11d 7'56.92"E, 55d 7' 4.83"N)
Lower Left ( 636000.000, 6108000.000) ( 11d 7'53.48"E, 55d 6' 0.18"N)
Upper Right ( 638000.000, 6110000.000) ( 11d 9'49.73"E, 55d 7' 2.84"N)
Lower Right ( 638000.000, 6108000.000) ( 11d 9'46.23"E, 55d 5'58.19"N)
Center ( 637000.000, 6109000.000) ( 11d 8'51.59"E, 55d 6'31.51"N)
Band 1 Block=256x256 Type=Byte, ColorInterp=Red
Band 2 Block=256x256 Type=Byte, ColorInterp=Green
Band 3 Block=256x256 Type=Byte, ColorInterp=Blue
Band 4 Block=256x256 Type=Byte, ColorInterp=Undefined